RamQuest 8/16

README.DOC

This file contains information, changes and enhancements

to the RamQuest 8/16 that doesn't appear in the user's manual.

Date: 06/14/90

---------------------------------------------------------------

Important information on the following topics is in this file:

1. - Release note

2. - MM.SYS command line switches/parameters

3. - Multi-Language support in INSTALL.EXE

4. - Using the RamQuest 8/16 in an OLIVETTI M290 system.

1. - Release note

Diskette Version 1.1 contains version 1.1 software:

MM816.SYS version 1.1:   supports up to 32 Mb Ems memory.

(8 4Mx9 simms installed)

INSTALL.EXE version 1.1: supports MM816.SYS version 1.1 and

French language.

The old MM.SYS and INSTALL.EXE should be removed and

replaced by the version 1.1 software from now on.

2. - Command line switches and parameters.

The syntax used looks like this:

DEVICE=[pathname]MM816.SYS /O=nnn /T /Z /F /A /S=nnnn /X=nnnn-nnnn ..

.. /H=nnn /C=nnn /D=nn

/O=nnn or /o=nnn

nnn refers to a 3 digit hexadecimal number which is the

I/O base address of the RamQuest 8/16's configuration

register.

Supported values for nnn:

208

300

308

310

600

608

Example: /O=208

/T or /t

Always test EMS memory.

/Z or /z

Don't test EMS memory.

/F or /f

Causes MM816.SYS to display internal data parameters.

A sample display:

Expanded Memory Manager Version 4.00

Copyright 1987,1990 Orchid Technology Inc. All Rights Reserved.

Copyright 1987,1990 Quadtel Corporation. All Rights Reserved.

Ramquest 8/16 Version XXX

EMS Window Address: D000

Contexts Avaliable: 255

Context Depth: 5

Handles Available: 255

Memory Avaliable: 4096

(Press ESC to skip memory test)

Testing Memory: 4096

The above was only an example; MM816.SYS will display

the appropriate values for your system's configuration.

/A or /a

INSTALL appends this switch to the command line of

MM816.SYS whenever exPanded memory is selected.

IBM has reserved the address range starting at 0E0000h and

ending at 0EFFFFh but some AT compatible makers have not.

This switch results in MM816.SYS searching

0C0000h to 0DC000h for an available exPanded memory

page frame and if unsuccessful, searching 0C0000h to 0EC000h

again before aborting exPanded memory driver installation.

If /A is not used, MM816.SYS will search 0C0000h to 0EC000h

for an available exPanded memory page frame.

/S=nnnn or /s=nnnn      (default value for /S is 0D000h)

nnnn refers to a 4 digit hexadecimal number which equals

a paragraph segment address at which MM816.SYS will place the

exPanded Memory Page Frame. The default is determined at

load time by MM816.SYS.

Supported values for nnnn:

C000

C400

C800

CC00

D000

D400

D800

DC00

E000

Example: /S=D000

/X=nnnn-mmmm or /x=nnnn-mmmm

Exclude a range of addresses from being used by MM816.SYS

as an exPanded memory page address.

nnnn refers to a 4 digit hexadecimal paragraph segment address

and mmmm refers to the ending range. nnnn is rounded down to

the nearest valid beginning range address and mmmm is rounded

up to the nearest ending range address.

Example: /X=C000-C7FF

This excludes the address space starting at 0C0000h

and ending at 0c7fffh. This address range is sometimes

used by VGA BIOS.

MM816.SYS uses several methods to detemine where it can place

the exPanded memory page address. However, not all devices

using a possible page address are detectable. This switch

can be used as a command to cause MM816.SYS to "skip" a

range of addresses.

/H=nnn or /h=nnn        (default value for /H is 255)

MM816.SYS uses a handle to associate with EMS page(s).

nnn is the maximum number of handles that can be used in

the system.

(3 minimum, 255 maximum)

/C=nnn or /c=nnn        (default value for /C is 255)

The number of contexts(3 minimum, 255 maximum)

/D=nn or /d=nn          (default value for /D is 5)

The depth of context saves(1 minimum, 32 maximum)

Note that the /H, /C, and /D options affect MM816.SYS's internal

memory buffer size.  MM816.SYS will allocate less Dos memory if

smaller values are set for /H, /C, and /D.

3. - Multi-Language support in INSTALL.EXE

Install.exe will now display messages/prompts in French

when it is started with the command line option 'F' or 'f':

e.g.:   Install f

4. - Using the Ramquest 8/16 in an OLIVETTI M290 system.

If extended memory is selected but the memory cannot be

recognized by the M290's boot up sequence, try to set the

starting address of Ramquest 8/16's extended memory to

1024K through the Install.exe.