[]---------------------------------------------[]

|						|

|	HGC 640/400 mono mode description	|

|						|

|		   Version 1.00			|

|	      Copyright (c) 1996 by		|

|		  Wojciech Migda		|

|						|

[]=============================================[]

HOW WAS IT INVENTED ?

~~~~~~~~~~~~~~~~~~~~~

Several times I was working on my old HGC trying to get more

than was offered in standard documentations hoping to get

better graphics. I Paid the main attention to CRT registers.

In all descriptions I've read was written that CRT register

number 8 MUST contain 02h. So I tried to write there something

else. Bingo! The magic number was 3. In text mode vertical size

of the screen was decreased and there were horizontal black

lines between every text line. I had to subtract 1 from the

letter size to get rid of them. In assembler it looks like

that:

mov	dx,03b4

mov	ax,0308

out	dx,al

mov	al,ah

inc	dx

out	dx,al

dec	dx

mov	ax,0c09

out	dx,al

mov	al,ah

inc	dx

out	dx,al

Done. Hmm.. we have the same number of lines like before and

a lot of free place above and below them. We could increase

the number of lines like with VGA card, but because of the

constant size of the memory page equal 1000h the screen is

being wrapped. If you know how to change it please tell me

(my adress at the end of this doc).

Then I tried to do the same in the graphics mode 720/348. It

changed the size of the screen too, but I couldn't recognize

the memory structure. Finally I discovered that like in the

text mode I needed to decrease content of the #09 register

which in text mode means height of letters but in graphics mode

it contains number of memory banks. The result was 640/400 mode.

It could have more lines but it is limited by the memory page

size equal 8000h, so maximum lines number is 409.

HOW TO TURN IT ON ?

~~~~~~~~~~~~~~~~~~~

Below are numbers (in hex) that must be written to CRT registers

to turn this mode on:

INDEX:    | 00  01  02  03  04  05  06  07  08  09

----------+----------------------------------------

CONTENTS: | 37  28  2d  07  68  1f  64  64  03  02

Memory is divided into 4 banks like in the 720/348 mode, but

the line size is equal 80 bytes. Every byte codes 8 pixels and

so on.

IS THIS CODE FREE ?

~~~~~~~~~~~~~~~~~~~

Yes, if the program which includes it is free too. Otherwise

you are obliged to send me 5 USD. Also nice postcards are

welcome.

HOW TO CONTACT THE AUTOR ?

~~~~~~~~~~~~~~~~~~~~~~~~~~

I can be reached under following adresses:

E-mail:

cooker@august.v-lo.krakow.pl

Snail-mail:

Wojciech Migda

os. Piastw 23/41

31-624 Krakw

POLAND