Over  the  past  few  years,   Intel  has  dominated  the  16-bit

microprocessor  market  with a device that has become one of  the

most popular microprocessors of all time,  the 8086 family, which

includes  the 8088/86/188/186/286 and the soon-to-come  80386,  a

mythical beast which IBM seems to be considering as the heart  of

their  next  PC.  The 286 and 386 microprocessors are awesome  on

their  own,  but it is just because of all that  available  power

that   many   hardware  and  software  manufacturers  are   still

attempting  to  squeeze  every  available  ounce  of  speed   and

throughput  out  of 8088/86 devices.  Too many people just  don't

know  exactly  how  much  the  80286  can  do,   and,  much  more

importantly,  how to get the device to do it.  In so many  words,

people  are  intimidated by these new and powerful  devices,  and

they  have decided to wait in the wings until someone figures  it

all  out  and  decides to share it with  us.  In  the  meanwhile,

software developers continue to attempt to squeeze their  devices

for every clock cycle they can get out of it.

Picture  this  scenario.  You are developing an  interrupt-driven

system for,  say,  a military avionics application, using a high-

level  language which until now has been ideally suited  for  the

application,  PL/M-86.  The device is a real-time terrain mapping

system, and the update and refresh timing specifications are very

tight,  20 milliseconds per frame or so.  You have worked for the

last  eight to ten months on developing the code for this system,

refining  it and optimizing it,  and inserting assembly  language

code  in  key places to give it that extra "oomph"  to  meet  the

timing  specifications.  You process the code,  load it into  the

system,  and begin checking your numbers to see if those last few

edits  did the job.  As you make your calculations and pound that

belligerent logic analyzer, it soon begins to dawn on you that no

matter what you do, you just can't get the code to fit the window

between  the  time your first screen refresh data  comes  in  for

processing,  and  the time three more sets of data are queued and

waiting  in the wings.  The processor just can't handle  the  job

fast enough, and you're not even working on worst-case situations

yet. Now what...

If  the CPU in question is either an 8088  or  8086,  relax,  our

friends  from  across the Pacific have come to  your  rescue.  It

seems that someone in Japan actually though about the phrase "New

and Improved",  so often found in our home TV sets,  and came  up

with   a   new  and  improved  version  of  the  8088  and   8086

microprocessors. How new and improved, you may ask? How does 100%

throughput performance improvement sound?  Good? Well, that's not

always the case,  but users of these new devices,  designated the

V20 and V30,  have reported such impressive numbers.  Too good to

be true,  you say?  Lean back in your chair, and listen to what I

have to say here. If you are using the Intel devices now, it will

be  a fresh of breath air for you,  but if  you  aren't,  proceed

anyway,  it's  fun reading,  and you just might switch after  you

finish it.

The  company  is  NEC,  and the name of the game is  to  make  it

faster,  better, and less power-hungry, but still compatible with

the old ones.  (Sounds familiar, doesn't it...) From the looks of

it, it seems NEC has achieved just that with their two new 16-bit

microprocessors,  the 70108D and 70116D,  better known as the V20

and  the V30.  These two devices are direct working  replacements

for the 8088 and 8086,  respectively, and are available in both 5

and 8mhz speeds, but what NEC has come out with is much more than

just  another  replacement  or  clone.  These  devices  are  high

performance,  enhanced microprocessors that offer a lot more than

just  compatibility.  In  this  article,  we will  explore  these

improvements and enhancements, and what the implications of these

are to those of us still using the 8088, 8086, 80188 and 80186.

First of all, the most obvious enhancement is the technology used

to  actually make the device.  The V20 and the V30 both are  CMOS

devices,  which  directly  translates to lower power  consumption

(300mw active) and less heat to dissipate inside your system.  If

you  have  ever  opened up your  16-bit  personal  computer,  you

may  have noticed that the 8088 and 8086 both generate a  lot  of

heat  and are very hot to the touch (if you touch them soon after

running them for a while...). These new devices run significantly

cooler,  and since they consume less power, reduce the demands on

your power supply.  Both of these chips have standby modes  also.

You  can  set the device into the standby mode,  while  retaining

interrupt service capability,  and during the period of time  the

device  is  in standby mode it will consume a tenth of the  power

required  in full active mode.  This feature comes very handy  in

laptop  and portable computers,  where power is a  very  precious

commodity.

Let's look a little deeper into the processor design and see what

new and exciting things we find.

The  most obvious change that stands out upon an initial scan  of

the  technical  specifications  is  that there  seem  to  be  new

registers and different names for the old ones.  Here's what they

are:

Mnemonic  Description                        8088/86 Equivalent

--------  ---------------------------------- --------------------

AW        Accumulator (16-bit)               AX

AH        High byte of AW                    AH

AL        Low byte of AW                     AL

BW        General register (16-bit)          BX

BH        High byte of BW                    BH

BL        Low byte of BW                     BL

CW        General Register (16-bit)          CX

CH        High byte of CW                    CH

CL        Low byte of CW                     CL

DW        General Register (16-bit)          DX

DH        High Byte of DW                    DH

DL        Low byte of DW                     DL

PC        Program counter                    IP

PS        Program Segment register           CS

SS        Stack Segment register             SS

DS0       Data Segment 0                     DS

DS1       Data Segment 1                     ES

PFP       Pre-Fetch Pointer                  No equivalent

SP        Stack pointer                      SP

BP        Base Pointer                       BP

IX        Index register (source)            SI

IY        Index register (destination)       DI

TA        Temporary register A (internal)    No equivalent

TB        Temporary register B (internal)    No equivalent

TC        Temporary register C (internal)    No equivalent

PSW       Program status word                Flag Register

The program status word is further subdivided into the  following

bit field definitions:

Bit       Description                        8088/86 Equivalent

--------- ---------------------------------- ------------------

0  : CY   Status  flag - Carry               CF

1  : --   reserved - pushed onto stack as 1  reserved

2  : P    Status  flag - Parity              PF

3  : --   reserved - pushed onto stack as 0  reserved

4  : AC   Status  flag - Auxiliary Carry     AUX

5  : --   reserved - pushed onto stack as 0  reserved

6  : Z    Status  flag - Zero                ZF

7  : S    Status  flag - Sign                SF

8  : BRK  Control flag - Break               TF

9  : IE   Control flag - Interrupt Enable    IF

10 : DIR  Control flag - Direction           DF

11 : V    Status  flag - Overflow            OF

12 : --   reserved - pushed onto stack as 1  reserved

13 : --   reserved - pushed onto stack as 1  reserved

14 : --   reserved - pushed onto stack as 1  reserved

15 : MD   Control flag - Mode                no equivalent/rsrvd

The  last  flag  has no equivalent in  the  8088/86,  because  it

controls a function that does not exist in those microprocessors,

8080 emulation mode.  Yes,  you did read that correctly.  More on

that later...

The  first design change item that stands out is a dual  internal

bus,  versus a single internal bus on the 8088 and 8086. Now that

integration  density  of  a  much higher order  of  magnitude  is

possible,  it is relatively inexpensive to integrate this feature

into  the device,  which is exactly what NEC has  done.  Internal

data  transmission within the device is now executed in parallel,

with both source and destination data traversing at the same time

on  the  full  16-bit wide busses.  This decreases  the  time  to

execute single instructions of any type.

The  8088  and  8086  calculate effective addresses  by  using  a

combination  of  hardware and  microcode,  essentially  a  little

program  within the device.  This process normally takes anywhere

between 5 and 12 clock cycles.  NEC has enhanced this part of the

microprocessor, replacing the microcode with a dedicated hardware

section which will perform effective address calculations of  any

type  in 2 clock cycles,  no more and no less.  Depending on  the

operation, this increases throughput in this area from 150 to 600

percent.

A completely new item added to the design is a Pre Fetch Pointer,

which works in conjunction with the instruction pointer.  The PFP

always  points to the next instruction to be executed,  no matter

how  close or far it may be from the  instruction  pointer.  This

feature  enhances  program  execution by allowing the  device  to

always  have  the  next  instruction  ready,  reducing  the  time

required to execute jumps, calls, returns, etc.

Three  new  16-bit  temporary registers have been  added  to  the

device,  used strictly for multiplication,  division,  shift  and

rotation  operation.  These  registers are NOT accessible by  the

programmer  or the application,  they are for internal use  only.

The significance of this is not fully realized until you actually

perform  some benchmarks,  and particularly when your  benchmarks

are  obtained  using math-intensive software.  I personally  have

performed  and witnessed some timing benchmarks on the  V30  that

were  absolutely  impressive.   Throughput  improvements  on  our

benchmarks,  consisting  of  math-intensive software  written  in

PL/M-86,  ranged between 66 and 82% increases.  Considering  that

the  only difference in the tests were the replacement of the CPU

in  the system,  from an 8086 to a V30,  we were quite  impressed

with  its  performance.  In addition to the temporary  registers,

there  is a Loop Counter,  which is used to count the  number  of

loops  for a primitive block transfer instruction and the  number

of   shifts   that   will  be  performed  for  a   multiple   bit

shift/rotation instruction.  This counter works very closely with

the  three  temporary registers,  and is a major  factor  in  the

improvement of processing speed.

The  V20 and V30 both implement an impressive 101 instruction set

which includes the 8088,  8086,  80188 and 80186 instruction sets.

In  addition,  it  also contains some new instructions  which  by

themselves  provide  tremendous  power and  flexibility  for  the

programmer.  There's one more instruction set available, but I'll

leave that for a little later.  Watch for it. First of all, let's

look  at the enhanced instructions that are available on the  V20

and V30.

Enhanced stack operation instructions

-------------------------------------

PUSH imm

--------

Allows immediate data to be pushed onto the stack

PUSH R

POP  R

------

These  two instructions allow the programmer to push the contents

of the 8 general registers (yes, all of them!) onto the stack.

Enhanced multiplication instructions

------------------------------------

MUL reg16,imm16

MUL mem16,imm16

---------------

These  instructions  allow the contents of a register  or  memory

location to be 16-bit multiplied by immediate data.

Enhanced Shift and Rotate instructions

--------------------------------------

SHL reg,imm8

SHR reg,imm8

SHRA reg,imm8

-------------

These instructions allow the contents of a register to be shifted

by the number of bits specified in the immediate data operand.

ROL reg,imm8

ROR reg,imm8

ROLC reg,imm8

RORC reg,imm8

-------------

Allow  for the same type of operation as above,  except  rotation

instead of shifting.

Check Array Boundary instruction

--------------------------------

CHKIND reg16,mem32

------------------

This  instruction is used to verify that index values pointing to

the  elements of an array data structure are within  the  defined

range.  The lower limit of the array should be in memory location

mem32,  the upper limit in mem32 + 2. If the index value in reg16

is not between these limits when CHKIND is executed, a BRK 5 will

occur.  This causes a jump to the location in interrupt vector 5.

This  instruction,  needless  to say,  is extremely powerful  and

could be a big time saver for programmers, but it will definitely

pose a problem for PC and compatible users, since their interrupt

5  executes  a  "print screen" routine.  This  problem  was  also

evident in the 80286 and the AT.

Block I/O instructions

----------------------

OUTM DW,src_block

INM  dst_block,DW

-----------------

These  instructions  are used to input or output a string  to  or

from  memory,  when preceded by a repeat  prefix.  Very  powerful

stuff.

Stack Frame instructions

------------------------

PREPARE imm16,imm8

------------------

This instruction is used to generate the stack frames required by

block-structured  languages,  such as PASCAL and Ada.  The  stack

frame  consists of two areas.  One area has a pointer that points

to  another frame which has variables that the current frame  can

access.  The  other  is  a local variable area  for  the  current

procedure.

DISPOSE

-------

This  instruction releases one from among the stack  frames  that

have  been generated by the PREPARE instruction.  It returns  the

stack and base pointers to the values they had before the prepare

instruction  was  used  to call a procedure.  Both this  and  the

PREPARE  instruction  are  very useful  to  compiler  developers,

saving quite a bit of time from having to code these functions.

The V20 and V30 also have a set of unique instructions which  are

not  available on the 8088 and 8086.  These instructions are very

powerful  and  useful in many  applications.  Support  for  these

instructions  is  available in the form of several compilers  and

development tools available for a variety of machines,  from  the

IBM-PC to the DEC VAX line of mainframes.

V20 - V30 Unique instructions

-----------------------------

Variable Length Bit Field operation instructions

------------------------------------------------

This  category has two instructions,  INS (Insert bit field)  and

EXT (Extract Bit Field).  These instructions are highly effective

for computer graphics and high-level language applications.  They

can,  for  example,  be used for data structures such  as  packed

arrays  and  record type data used in  PASCAL.  CAD/CAM  software

should  be  able  to benefit immensely from  these  instructions,

which could quite easily reduce code size by anywhere from 10  to

30% and increase performance tremendously.

INS reg8,reg8

INS reg8,imm4

-------------

These  instruction transfer low bits from the 16-bit AW  register

(the  number  of bits is specified by the second operand) to  the

memory location specified by the segment base (DS register)  plus

the  byte offset (IY) register.  The starting bit position within

this  byte  is specified as an offset by the lower 4-bits of  the

first operand. After each complete data transfer, the IY register

and  the  register  specified by the lower 4 bits  of  the  first

operand  are  automatically updated to point to  the  next  index

field.  Either  immediate data or register may specify the number

of  bits  transferred  (second  operand).   Because  the  maximum

transferable bit length is 16-bits,  only the lower 4 bits of the

specified register (00H to FFH) will be valid. Bit field data may

overlap the byte boundary of memory.

EXT reg8,reg8

EXT reg8,imm4

-------------

These  instructions  load to the AW register the bit  field  data

whose  bit field length is specified by the second operand of the

instruction from the memory location that is specified by the DS0

segment  register  (segment base),  the IX index  register  (byte

offset),  and the lower 4 bits of the first operand (bit offset).

After the transfer is complete,  the IX register and the register

specified  by  the  lower  4  bits  of  the  first  operand   are

automatically  updated  to point to the next  bit  field.  Either

immediate  data  or  a register maybe specified  for  the  second

operand.  Because the maximum transferable bit length is 16 bits,

however,  only  the lower 4 bits of the specified register (0H to

0FH) will be valid.  Bit field data may overlap the byte boundary

of memory.

Packed BCD Operation instructions

---------------------------------

These instructions provide the programmer with valuable tools  to

handle  packed binary-coded-decimal (BCD) operations  within  the

application  being  developed.  The  BCD format is most  used  in

business applications,  where numeric precision is a requirement.

The instructions described here process packed BCD data either as

strings  (ADD4S,  SUB4S,  CPM4S) or byte-format  operands  (ROR4,

ROL4).  Note  that to perform string processing for the  rotation

instructions,  the  programmer  must use the  macro  instructions

ROR4S  and  ROL4S,  available with NEC's language  and  assembler

support software.

ADD4S

-----

This  instruction adds the packed BCD string addressed by the  IX

index register to the packed BCD string addressed by the IY index

register, and stores the result in the string addressed by the IY

register.  The  length  of the string (number of BCD  digits)  is

specified  by  the CL register,  and the result of the  operation

will affect the overflow flag (V),  the carry flag (CY),  and the

zero flag (Z).

SUB4S

-----

This instruction subtracts the packed BCD string addressed by the

IX  index register from the packed BCD string addressed by the IY

register, and stores the result in the string addressed by the IY

register.  The  length  of the string (number of BCD  digits)  is

specified  by the Cl register,  and the result of  the  operation

will  affect the overflow flag (V),  the carry flag (CY) and  the

zero flag.

CPM4S

-----

This  instruction  performs the same operation as  SUB4S,  except

that  the result is not stored,  and only the overflow flag  (V),

carry flag (CY) and zero flag (Z) are affected.

ROL4

----

This  instruction  treats  byte data of the  register  or  memory

directly  specified by the instruction byte as BCD data and  uses

the  lower 4 bits of the AL register to rotate that data one  BCD

digit to the left.

ROR4

----

This  instruction  works exactly like the ROL4,  except  the  end

result is a shift of one BCD digit to the left.

Bit Manipulation Instructions

-----------------------------

This  instructions  allow the programmer to  manipulate  distinct

bits within registers or memory locations,  instead of having  to

use  masks,  AND's,  OR's,  etc.  They are very useful in bit-map

graphics-oriented  and other similar applications.  Use of  these

instructions should decrease code size considerably.

TEST1

-----

This  instruction  tests a specific bit in a register  or  memory

location.  If the bit is 1,  the Z flag is reset to 0. If the bit

is 0, the Z flag is set to 1.

NOT1

----

This  instruction inverts a specific bit in a register or  memory

location.

CLR1

----

This  instruction clears a specific bit in a register  or  memory

location.

SET1

----

This  instruction  sets  a specific bit in a register  or  memory

location.

Repeat Prefix Instructions

--------------------------

These  instructions  allow for high speed block  primitive  block

transfers  to  be executed repeatedly based on the  status  of  a

single flag, the CY flag.

REPC

----

This   instructions  causes  the  microprocessor  to  repeat  the

following primitive block transfer instruction until the CY  flag

becomes cleared.

REPNC

-----

This   instruction  causes  the  microprocessor  to  repeat   the

following  primitive block transfer instruction until the CY flag

becomes set.

Floating Point Operation Instructions

-------------------------------------

FPO1 fp-op,mem

FPO2 fp-op,mem (new)

--------------------

These  instructions  are  used for the  external  floating  point

processor. The floating point operation is passed to the floating

point  processor when the CPU fetches one of these  instructions.

From  this  point the CPU performs only the  necessary  auxiliary

processing (effective address calculation, generation of physical

addresses, and start-up of memory read cycle). The floating point

processor  always  monitors the instructions fetched by the  CPU.

When  it interprets one as an instruction to itself  it  performs

the  appropriate  processing.  At this time,  the floating  point

processor chip uses either the address alone or both the  address

and read data of the memory read cycle executed by the CPU.  This

difference   in   the  data  used  depends  on  which  of   these

instructions is executed.

Standby Function

---------------

The  V20  and  V30  both  have a standby  mode  to  reduce  power

consumption during wait states,  a mode which comes in very handy

when  designing systems that run on batteries or some other  type

of limited power source. This mode is set by the HALT instruction

in both the native and emulation mode.  In the standby mode,  the

internal  clock  is supplied only to those  circuits  related  to

functions  required  to  release this mode and bus  hold  control

functions.  As  a result,  power consumption can be  reduced   to

1/10th the level of normal operational requirements in either the

native  or  emulation  modes.  The standby mode  is  released  by

inputting  a RESET signal,  or an external interrupt (NMI,  INT).

The bus hold function is effective during standby mode.  The  CPU

returns to standby mode when the bus hold request is removed.

Mode Operation Instruction

--------------------------

At  the beginning of this article I made a few references to  the

mode  flag,  an emulation mode,  and a few other items related to

this subject.  All these references are related to a mode of this

microprocessor which none of the Intel processors have.  The  V20

and   V30   are  capable  of  fully  emulating  the  8080   8-bit

microprocessor,  including  its full instruction set.  There  are

four instructions directly related to this emulation mode.

BRKEM imm8

----------

This  is the basic instruction used to start the  8080  emulation

mode.  This  instruction  operates  exactly the same as  the  BRK

instruction,  except  that BRKEM resets the mode flag (MD) to  0.

The PSW,  PS and PC registers are saved onto the stack, and MD is

then reset and the interrupt vector specified by the operand imm8

of this command is loaded into PS and PC.  The instruction  codes

of  the interrupt processing routine jumped to are then  fetched.

The CPU then executes these codes as 8080 instructions.

The registers and flags of the 8080 are mapped onto the V20 - V30

registers and flags as follows:

8080 Register  V20 - V30 Register

-------------  ------------------

A              AL

B              CH

C              CL

D              DH

E              DL

H              BH

L              BL

SP             BP

PC             PC

8080 Flags     V20 - V30 Flag

-------------  ------------------

C              CY

Z              Z

S              S

P              P

AC             AC

The  use of independent stack pointers (SP in native mode,  BP in

emulation  mode)  allows separate stack areas to be  defined  for

each  mode  and keeps the stack of one of the  modes  from  being

destroyed by stack operations in the other mode.

The native-mode SP,  IX,  IY and AH registers are not affected by

any operations executed during emulation mode.

In the 8080 emulation mode,  the code segment register is defined

by  the  PS register (set by the interrupt vector) and  the  data

segment  register if defined by the DS0 register.  This last item

must  be  set  by  the programmer  immediately  before  the  8080

emulation mode is entered.

RETEM

-----

When RETEM is executed in the 8080 emulation mode (interpreted by

the CPU as an 8080 instruction),  the CPU restores the PS, PC and

PSW (just like if it were returning from an interrupt  processing

routine),  and returns to the native mode.  At the same time, the

contents  of the mode flag (MD),  which was saved to the stack by

the BRKEM instruction,  is restored to a value of 1.  The CPU  is

now  set to native mode,  and executes all following instructions

as normal V20 - V30 instructions.

CALLN imm8

----------

This   instruction  makes  it  possible  to  call   native   mode

subroutines  while  in  8080 emulation mode.  To  return  from  a

subroutine  to the 8080 emulation mode,  the RETI instruction  is

used.  In  this  manner,  system  calls that  would  normally  be

serviced by 8080 subroutines in a CP/M environment,  for example,

can  be rerouted to native mode service routines,  without having

to develop more code to take care of these situations.  It  would

require a lot of ingenious programming,  but the tools to do this

are available here.

The processing performed when this instruction is executed in the

8080  emulation  mode is similar to that performed when a BRK  is

executed  in  the  native mode.  The imm8  operand  specifies  an

interrupt  vector type.  The contents of the PS,  PC and PSW  are

pushed  onto the stack and an MD flag value of 0  is  saved.  The

mode flag is set to 1,  and the interrupt vector specified by the

operand is loaded into the PS and PC.

RETI

----

This  is  a  general  purpose instruction  used  to  return  from

interrupt  routines  entered  by  the BRK instruction  or  by  an

external interrupt in the native mode.  When this instruction  is

executed at the end of a subroutine entered by the execution of a

CALLN instruction, the operation that restores the PS, PC and PSW

is restored,  however,  the 8080 emulation mode value of the mode

flag (MD) is restored, the CPU is set in emulation mode, and 8080

emulation is resumed at that point.

As  you  can see,  NEC has implemented very useful and  practical

instructions  and added them to an already  powerful  instruction

set. The instructions that they have chosen to add and/or enhance

can  be  used to reduce code and/or decrease processing time  for

just about any software designed to run on the 8086, 8088, 80188,

80186, and even the 8080 microprocessor.

This  device has been installed as a replacement to the 8088  and

8086  in  many systems,  ranging from single board  computers  to

full-fledged mini- and microcomputers,  and it has worked fine so

far.  For you IBM-PC and compatible users,  the only problems  we

have  encountered have been with software that is  copy-protected

and  uses  exotic  schemes to determine whether or  not  you  are

running  an authorized copy of the package,  or whether you  have

your  key  disk inserted in the main floppy disk drive.  In  some

cases,  you  can  use the commercially  available  software  that

allows you to bypass the check for the key disk in drive A:. This

is  NOT an encouragement to bypass copy-protection,  it is only a

suggestion  to  legal owners of this type of software  who  might

have  problems  of this type.

Purchase  of  this  device is an option that  many  microcomputer

owners might want to consider as an add-on that will not take  up

any  precious  expansion slots and will enhance  productivity  on

their systems.  It is relatively easy to install in your machine,

depending  on  the  type and  configuration,  and  is  relatively

inexpensive (about $50 retail,  compared to anywhere from $800 to

$1300  for a typical co-processor board).  Implementation of  the

new and enhanced instructions at an assembly language level could

be achieved with the addition of a set of macros to automatically

translate  the opcodes into the machine language the machine will

understand.

##########################################################################################################################  point

processor. The floating point operation is passed to the floating

point  processor when the CPU fetches one of these