* * * * * * UPDATED FEATURES GENIUS DRIVER V9.04 * * * * * *

The following mouse parameters have been added to the GMOUSE

driver which allow you to configure the mouse or trackball to

fit your requirements.

A. Dynamic Resolution ( *m )

-------------------------

With this parameter, an exciting new concept in Mouse control

has been developed.  A precise location on the screen can be

selected with greater ease  and speed.

Movement of the cursor across the screen is based on the tra-

vel count and speed of the movement of the mouse.  It is not

only measured the distance traveled by the mouse, but also

by checking how quick the mouse is moved. The faster the

movement of the mouse allows the cursor to move farther

across the screen. This action causes less hand movement.

The resolution of the input device (mouse or trackball) can

be increased up to 1400 dots per inch depending on the

device you are using.  You can select the desired resolution

according to the requirements of the software program

installed.

To select the resolution, type the command:

A >GMOUSE *m   (Where m = 1 to 4, if omitted, default is 2)

B. Reduce Resolution ( /d )

------------------------

The resolution of the input device (mouse or trackball) can

also be reduced if you require a lower resolution than the

one that is currently being used.

To reduce the resolution, enter the following command:

A >GMOUSE /d   (Where d = 2 to 4, and the number entered

divides the current resolution by this amount).

C. Communication Port ( n )

------------------------

There are four general purpose communication ports generally

used on your personal computer. The relative address and IRQ

no. are listed in the table below:

-------+----------+------------------+----------

n   |  COM no. |  Address in HEX. |	IRQ no.

-------+----------+------------------+----------

1   |  COM 1   |	  3F8	     |	IRQ 4

2   |  COM 2   |	  2F8	     |	IRQ 3

3   |  COM 3   |	  3E8	     |	IRQ 4

4   |  COM 4   |	  2E8	     |	IRQ 3

-------+----------+------------------+----------

The Genius input device (mouse or trackball) can be set to

any one of the four communication ports, (COM1 to COM4). The

command format is:

A >GMOUSE n    (Where n = 1 to 4).

For example, if you install the mouse at COM1 (default

setting), enter the following: A >GMOUSE 1 or A >GMOUSE

And if you want to install the mouse at COM4, enter the

following:	   A >GMOUSE 4

Follow the above examples if your communication port is

different than the above examples.

D. I/O Port Address & IRQ no. ( -a,i )

-----------------------------------

This new feature is provided for experienced users. If you're

unfamiliar with I/O ports and IRQ channels, don't attempt to

change these because any mistakes could prevent you from

using your computer. You change the addresses when the I/O

port for the Genius input device (mouse or trackball) is

different from the default settings as described in the

table above. To change the port address and IRQ channel,

follow the steps listed below to select hardware interrupt

(IRQ no., range from 2 to 5, the preset IRQ no. for user as-

signable port address is 5) and the I/O port address to

install the mouse driver.

1). To select the I/O port address and IRQ no., enter:

A >GMOUSE -3E0,2 (Setup driver, select 3E0H port address

and IRQ no. 2)

2). To select the I/O port address and preset IRQ no.5,

enter:

A >GMOUSE -260   (Setup driver, select 260H port address

and preset IRQ no. 5)

WARNING: If the wrong setting is selected or not set

properly, you may cause system error!

E. Switch Ortho line Function  ( +s )

----------------------------------

The Ortho line Function is provided as an aid to the user

to draw straight horizontal or vertical lines. You can

set this function to be ON or OFF, by setting the parameter

to "+1 or +0". The default for this function is OFF.

Once you have set the Ortho line function "ON", you press

the left and right button of the input device (mouse or

trackball) at the same time to start the function. Use the

same method to turn the function "OFF" when you don't want to

use it.

To activate the Ortho line Function, type the command:

A >GMOUSE +1   (Switches the Ortho Line Function ON)

A >GMOUSE +0   (Switches the Ortho Line Function OFF)

F. Multiple Parameters

------------------

It is also possible to select the resolution and the commun-

ication port at the same time. The sequence of selecting the

multiple parameter is:

1). Resolution (*m or /d)

2). Ortho line Function (+s)

3). Communication port (n or -a,i)

Where m is from 1 to 4, /d is from 2 to 4, n is from 1 to 4,

-a is the port address, and i is the IRQ no.

Ex: A >GMOUSE *4+1-3E8,4

This case allows for faster movement of the cursor (*4) on

the screen and the mouse port address is 3E8H which IRQ4 is ass-

igned. We also activated the Ortho line Function (+1).

Ex: A >GMOUSE *3-279

The I/O address is 279H and this command uses the default

IRQ no. 5.

Ex: A >GMOUSE +1-260

The Ortho line function is activated and we set the I/O port

address to 260H also uses the default IRQ no. 5.

Ex: A >GMOUSE /22

This command connects your mouse to communication port COM2

and the movement of the cursor becomes slower (cursor

movement is reduced by 1/2 of the current amount).

Ex: A >GMOUSE 4

This command connects your mouse to communication port COM4

and uses the address and IRQ no. associated with COM 4 (refer

to the table in section C).

G. How to use GMOUSE.SYS file

--------------------------

The Genius mouse driver (also used for the trackball) comes

with two different file formats; one in the command format

named GMOUSE.COM, and the other in DOS system format named

GMOUSE.SYS.  These two file functions are exactly the same.

The only difference is the GMOUSE.COM file can be executed at

the DOS prompt and the GMOUSE.SYS must be placed in your

system's CONFIG.SYS file. You can used either file

depending on your preference whether or not you want the

mouse driver to be installed in your CONFIG.SYS. To use the

GMOUSE.SYS file, insert the line "DEVICE = GMOUSE.SYS

+1-3E8,2" in the CONFIG.SYS file.

Note: The GMOUSE.SYS and CONFIG.SYS files must be at the root

directory of the disk which is used to start DOS. The

mn parameters were added to choose the GMOUSE driver's

resolution and communication port. These were

described in the sections A,B,C, and F. Section D is

for experienced users and section D described the Ortho

line function.

H. The Genius Driver V9.04 provides a bus mouse interface which

is suitable for Microsoft Bus Mouse, Microsoft InPort Mouse,

and Genius Bus Mouse Versions: GM-7, GM-7A, and GM-7D.

Important: This README.DOC takes precedence over the user's

manual that accompanies your input device (mouse

or trackball). The Genius driver V9.04 is the

latest driver currently available. The driver is

continually updated and if you're trying to run

a new software program and having problems using

the input device, please contact the place of

purchase for assistance.