(CORRECTED +ODDSEGMENT output on 74f04 --- should be pin10)

Hank - In case this works, I give a wordy intro in case anyone wants to

try the same thing.  Maybe you can provide some insight and improvement.

Also, please write down the 64k chip data before you piggyback them...

it might help in debugging for you or someone else.  GOOD LUCK!

HOWTO: Upgrade a PCjr Memory sidecar to 384K

BY: Dave CONTOIS at KGNVME

These instructions are for upgrading an IBM 128K sidecar to 384K by

adding a single bank of (8) 256k chips and (16) 64k chips; the

existing 64k chips can be reused if you're careful (you may want to

refer to the heat gun technique in PCJRSIDE FORUM).  Two TTL chips

are also piggybacked in the spare 14 pin location on the card; these

chips are needed to separate the RAS signal to Bank 0 into two lines

(RAS00 and RAS01) and provide refresh to the chips.  Two resistors

are also required (to terminate RAS00 and RAS01); 30 to 50 ohm,

1/4 or 1/2 watt should be fine.

These instructions have NOT been tested, use at your own risk!

Remove the existing 64k chips from the system board and install

16 pin sockets.  This would be a good time to install a 14 pin

socket in the unused location to the right of the 48 pin DRAM

controller.

Install (16) 64k chips in the sockets; if you reuse the chips you

removed, clean and straighten the pins first.  Now ensure the

sidecar still works.... If there's a problem, JRMD and an ohmmeter

will usually track it down.  AFTER the sidecar works, remove it and

unplug the 64k chips.

Next, "piggyback" one chip on top of another (make SURE pin 1 is the

same on both) and (carefully) bend pin 4 of the top chip into a

shallow "U".  Now solder pin 1 of the top chip to pin 1 of the

bottom chip, being careful to keep solder off the bottom of the

lower pin (we want the pins to plug into the sockets later!)

Continue until all pins except pin 4 are soldered together.

Now do all that SEVEN MORE TIMES.  (How much are 256k chips?)

(baloney ends, real meat begins)---djc

BOARD WIRING

Cut trace and add jumper per Crumley's instructions.

Follow -RAS0 from '409 to SIP resistor; follow through resistor

and cut trace on top of board between resistor and via (we will

be attaching a resistor/wire to the via later).

pin28  o-----------o/\/\/\o------ CUT ----------o

l

Add wire from cut end of   l  resistor to spare socket pin 3 (s3)

l

l-----------------o  s3

Add wire from '409 pin29 (-RAS1) to s1.

Find Address16; add wire from A16 to s13.

Add wire from s12 to s11.

Add wire from s11 to s5.

Add wire from s10 to s9.

End Board Wiring.

CHIP WIRING

Piggyback a 74F51 on top of a 74F04.  Ensure pin 1's line up!

Solder pin 2 top to bottom; repeat for pin 4,5,7,9,10, and 14.

On the 74F51 (TOP) chip:

Bend pins 6 and 8 nearly horizontal; resistors will be soldered

to them later.

Bend up pins 1,3,11,12, and 13.

Solder pin 1 to pins 2 and 13.

Solder pin 3 to pins 4, 11, and 12.

Plug into spare socket (Pin 1 is lower left?)

Plug stacked chips in Bank 0 and 256k chips in Bank 1 :

01 0101010101 0101 ???? (check layout in JRMD)

Wire all pin 4's of the upper 64k chips together; the wire should be

mechanically rigid and should not touch the case or other components.

Wire a resistor from 74f51 pin6 to the via near where -RAS0 was cut.

I call this "-RAS00".

Wire a resistor from 74f51 pin8 to the middle of the "pin4 bus".

I call this "-RAS01".

END Chip Wiring.

TESTING

Set switches 1,2,3 ON, 4 OFF.

Plug in and test.

Even if it POSTs to 512K, you should run the JRMD data tests

with the REFRESH test option (wait at least 2 minutes...I want

to be SURE the refresh is working properly.

DATA

ADDRESS BITS   SEG  128K  512K  384K(TEST)

19 18 17 16         (IBM SIDECAR BANKS)

-------------------------------------------

0  0  0  0     0   XXXXXXXXXXXXXXXXXXX (PCJR SYSTEM BOARD AND

0  0  0  1     1   XXXXXXXXXXXXXXXXXXX  64K EXPANSION CARD MEMORY)

0  0  1  0     2    0     0    00+ \

0  0  1  1     3    1     0    01+ / SWITCH 1

0  1  0  0     4    0     1     1  \

0  1  0  1     5    1     1     1  / SWITCH 2

0  1  1  0     6    0     1     1  \

0  1  1  1     7    1     1     1  / SWITCH 3

1  0  0  0     8    0     0     *  \

1  0  0  1     9    1     0     *  / SWITCH 4

+ The added logic decodes bank0 and bit16 (segments 2 and 3)

and creates bank00 and bank01 select lines for the stacked

64k ram chips.

* SEGMENTS 8 AND 9 DO NOT EXIST on the 384K modification;

as designed, switches 1, 2, and 3 MUST be ON and

***********  switch   4  *******  MUST be OFF! ********

DIAGRAM               74F04 Inverter

__________

l         l

pin29_____-RAS1_____1 l         l 2_______+RAS1

l         l

pin28_____-RAS0_____3 l         l 4_______+RAS0

l         l

???____ADDR16______13 l         l 12______+EVENSEGMENT

l         l

____+EVENSEGMENT___11 l         l 10______+ODDSEGMENT

l_________l

DIAGRAM               74F51 Dual And/Or/Invert Gates

__________________________

l                          l

+RAS1__________2 l A \                      l

l N  l-l                   l

+RAS0__________3 l D /  l---\               l

l       l           OR---INVERT----l 6________-RAS00

l_____4 l A \  l---/               l

l N  l-l                   l

+EVENSEGMENT___5 l D /                      l

l__________________________l

__________________________

l                          l

+RAS1_____o____1 l A \                      l

l---13 l N  l-l                   l

+RAS0_________12 l D /  l---\               l

l       l           OR---INVERT----l 8________-RAS01

l____11 l A \  l---/               l

+ODDSEGMENT_o_10 l N  l-l                   l

l--9 l D /                      l

l__________________________l

Logic Description:

RAS00 is active (minus) if either statement is true:

RAS1 and RAS0 are both active (indicates DRAM REFRESH cycle)

RAS0 is active and address bit 16 is off (indicates a memory

cycle for segment 2)

RAS01 is active (minus) if either statement is true:

RAS1 and RAS0 are both active (DRAM REFRESH cycle as above)

RAS0 is active and address bit 16 is on (indicates a memory

cycle for segment 3)

(Hank Kennedy only....home phone (914)876-3356....IF you get to it

this weekend, AND want to talk...feel free!  Otherwise, c u next week)