### PCD-47B Firmware Release Note

V3.1 Released on May 10, 2001

File:  CARD_PCA.BIN 131,072 Bytes, 03/01/01 03:01AM

Bug fixes:

- This release fixes a bug in the firmware that wrote to the Configuration Register (at Offset 0x200) a value of 0x01 after issuing an ATA IDENTIFY (0XEC) command. This caused problems in Mitsubishi 7.5MB and 15MB media.

- This release also corrects a rare instance of one or multiple drives not being scanned and identified during the SCSI enumeration process.  This caused the drive(s) to not appear in Windows Explorer/My Computer.

Feature additions:

- None.

Notes:

- This firmware is available as a BIN file which can be downloaded into an existing drive using the Firmware Download utility. The BIN file in Rx.x folder is the “Read Only” Firmware in which the media is always reported as a write protected media. The BIN file in Mx.x folder is the normal firmware in which media can be read and written.