������������������������������������������������������������������������������

۱������������                                                  ް������������

۰������������              ������  "MODEM"  �����              ޱ������������

۱������������                                                  ް������������

۰������������               by Pinnacle Software               ޱ������������

۱������������                                                  ް������������

۰������������              A Modem Control Module              ޱ������������

۱������������                                                  ް������������

۰������������                     �� FOR ��                    ޱ������������

۱������������                                                  ް������������

۰������������      Batch Files  and  Application Programs      ޱ������������

۱������������                                                  ް������������

������������������������������������������������������������������������������

MODEM  enables you to perform basic modem operations within a batch file,  and

check the  results  using  DOS ERRORLEVEL  processing.   A high-level language

(such a C, Basic or Pascal) can "shell" or "exec" MODEM  and check the results

via the DOS return code.

Thus, you can:

����������������������������������������������������������������������������Ŀ

�                                                                            �

�  � Automate modem-oriented operations such as late-night file-transfers    �

�                                                                            �

�  � Easily add the benefits of modem control to your application program    �

�                                                                            �

�  � Run MODEM as a simple terminal program                                  �

�                                                                            �

�  � Test your serial ports or modem with the TRACE facility                 �

�                                                                            �

�  � Use MODEM as an auto-dialer                                             �

�                                                                            �

�  � Support BBS operations                                                  �

�                                                                            �

�  � and more ...                                                            �

�                                                                            �

������������������������������������������������������������������������������

Using MODEM in a batch file,  along with an external protocol such as the Omen

Technology's  DSZ program,   you can  easily  create  your own  communications

program, with upload and download capbility.

������������������������������������������������������������������������������

۱������������                                                  ް������������

۰������������                 QUICK REFERENCE                  ޱ������������

۱������������                                                  ް������������

������������������������������������������������������������������������������

Usage:     Enter the  MODEM  command at  the DOS prompt,  call it from a batch

file, or shell or exec it from an application program.

Format:    MODEM port command   (See also Extended Support for another format)

Example:   MODEM 1 TERM

Commands:  MODEM port SETBAUD baudrate

MODEM port GETBAUD

MODEM port ATcommand

MODEM port WAIT ATcommand [greeting-string]

MODEM port DIAL ATcommand

MODEM port RAMDIAL ATcommand

MODEM port CARRIER

MODEM port DSR

MODEM port DROPDTR

MODEM port PAUSE time-in-seconds

MODEM port TERM

MODEM port TRACE

������������������������������������������������������������������������������

۱������������                                                  ް������������

۰������������                 MODEM  COMMANDS                  ޱ������������

۱������������                                                  ް������������

������������������������������������������������������������������������������

��������������������������������������������������������������Ŀ

� NOTE: The examples shown below refer to COM1 and COM2 -- see �

� the section "Extended Support" for notes about COM3 and COM4 �

����������������������������������������������������������������

Enter one of the following commands at the DOS prompt...

�����������������������������������������������������������������������������

Command:  MODEM port SETBAUD baudrate

Example:  MODEM 2 SETBAUD 1200

Action:   Initializes modem port and sets it to the specified baudrate.

Use this command before any of the other commands so that your

computer's serial port will know what speed it should be at.  If

your modem acts oddly afterwards, you may have to turn it off and

on and issue an AT command so that it can figure out what the baud-

rate of the serial port is.  External modems typically do NOT have

any way of figuring out the serial port speed except by looking at

the first characters received and trying to figure out what speed

would make them look like "AT".

Returns:  0

�����������������������������������������������������������������������������

Command:  MODEM port GETBAUD

Example:  MODEM 1 GETBAUD

Action:   Displays the baud rate that the port is set to.

Returns:  11 =  300 baud; 12 = 1200 baud; 13 = 2400 baud; 14 = 4800 baud;

15 = 9600 baud; 16 = 19200 baud; 17=38400; 25=Over 38400

�����������������������������������������������������������������������������

Command:  MODEM port ATcommand

Example:  MODEM 1 ATH1

Action:   Sends AT command to the modem and waits for OK.

Returns:  0 if OK; 200 for any other response

�����������������������������������������������������������������������������

Command:  MODEM port WAIT ATcommand [greeting-string]

Example:  MODEM 2 WAIT ATS0=1

Action:   Sends AT command to the modem and waits for an incoming call.

Returns:  11 =  300 baud; 12 = 1200 baud; 13 = 2400 baud; 14 = 4800 baud;

15 = 9600 baud; 16 = 19200 baud; 255 if user pressed ESC to cancel

Notes:    19200 baud does not function reliably on 4.77 Mhz machines.

The greeting-string, if specified, is transmitted at connect time.

�����������������������������������������������������������������������������

Command:  MODEM port DIAL ATcommand

Example:  MODEM 1 DIAL ATDT1-514-345-8654

Action:   Sends AT command to modem and waits for connection to remote system.

Returns:  11 =  300 baud; 12 = 1200 baud; 13 = 2400 baud; 14 = 4800 baud;

15 = 9600 baud; 16 = 19200 baud; 255 if user pressed ESC to cancel

�����������������������������������������������������������������������������

Command:  MODEM port RAMDIAL ATcommand

Example:  MODEM 1 RAMDIAL ATDT1-514-345-8654

Action:   Same as DIAL, but dials until connection, or until ESC is pressed.

�����������������������������������������������������������������������������

Command:  MODEM port CARRIER

Example:  MODEM 2 CARRIER

Action:   Checks for presence of carrier.

Returns:  0 if carrier present; 1 if not present

�����������������������������������������������������������������������������

Command:  MODEM port DSR

Example:  MODEM 1 DSR

Action:   Checks for presence of dataset ready (modem turned on).

Returns:  0 if ready; 1 if not ready

�����������������������������������������������������������������������������

Command:  MODEM port DROPDTR

Example:  MODEM 1 DROPDTR

Action:   Lowers Data Terminal Ready.  This hangs up most brands of modem,

though you should follow up with a "PAUSE" command to give the

modem time to complete the disconnect.

�����������������������������������������������������������������Ŀ

�� VERY IMPORTANT NOTE:   If you are using a modem with built-in ��

�� error correction or compression, dropping DTR does not ALWAYS ��

�� hang up the modem -- if the protocol negotiation fails.  This ��

�� exquisite little bug in the firmware of some modems  has cost ��

�� some companies a lot of money in  long-distance bills.  Don't ��

�� let it  catch you!   When in doubt,  follow up DROPDTR with a ��

�� PAUSE, then ECHO +++ >COM1, another pause, then ATZ.          ��

�������������������������������������������������������������������

You can restore DTR by issuing almost any other MODEM command, such

as setting the baud rate, entering TERM mode, etc.

Returns:  0

�����������������������������������������������������������������������������

Command:  MODEM port PAUSE time-in-seconds

Example:  MODEM 2 PAUSE 15

Action:   Monitors the modem for incoming characters, which are displayed,

until the time is up or the user presses spacebar or ESC.

Returns:  0 for time-out; 254 if user hit spacebar; 255 if user hit ESC

(Some alternative pausing utilities are available from Pinnacle

in our "GO" package.  Call us at 514-345-9578 for details.)

�����������������������������������������������������������������������������

Command:  MODEM port TERM

Example:  MODEM 1 TERM

Action:   A simple terminal program for checking and setting modem parameters.

Returns:  0

�����������������������������������������������������������������������������

Command:  MODEM port TRACE

Example:  MODEM 1 TRACE

Action:   Similar to TERM, but displays control codes.

Returns:  0

�����������������������������������������������������������������������������

������������������������������������������������������������������������������

۱������������                                                  ް������������

۰������������                PROGRAMMING NOTES                 ޱ������������

۱������������                                                  ް������������

������������������������������������������������������������������������������

Error Levels

������������

Modem generates an ERRORLEVEL of 255 if the program aborts due to a detectable

program or hardware flaw, an invalid parameter,  or some other unusual circum-

stance.

Dialog

������

To send text to the modem, you can use DOS's ECHO command, as in this example:

ECHO Hello World! >COM1

If you require more sophisticated dialog support,  contact  Pinnacle  Software

for information about a variety of options open to you.

������������������������������������������������������������������������������

۱������������                                                  ް������������

۰������������              ENVIRONMENT VARIABLES               ޱ������������

۱������������                                                  ް������������

������������������������������������������������������������������������������

For testing purposes,  you may want the capability to bypass the MODEM command

and simply  force it to return an ERRORLEVEL of  0.   To  do  this,  enter the

following DOS command:

SET MX=L

Afterwards, to restore MODEM to normal operation, enter this command:

SET MX=

������������������������������������������������������������������������������

۱������������                                                  ް������������

۰������������                EXTENDED  SUPPORT                 ޱ������������

۱������������                                                  ް������������

������������������������������������������������������������������������������

The general format of the Extended Support MODEM commands is:

Format:   MODEM [extended-support-parameters] port command

Example:  MODEM /L /I3 4 TRACE

Action:   Runs a logged trace on COM4, using the IRQ3 interrupt.

Extended Support parameters are optional.   They may begin with a slash (/) or

a dash (-).

You are entitled to use the basic version of MODEM  completely free of charge.

The  following features may be obtained by purchasing the latest copy of MODEM

from  Pinnacle  Software.  (Note:  when you order your copy,  we'll send you a

large collection of  modem-oriented software - see the order form for complete

details.)

�����������������������������������������������������������������������������

Command:  MODEM /I# port command

Example:  MODEM /I3 3 TERM

Action:   Allows you to specify the IRQ line used by the COM port.  The

Extended Support version of MODEM lets you use COM3 and COM4, but

some modems do not use the generally used values for these ports,

which are:

���������������������������������������������������������������͸

�               �               �               �               �

�  COM1 = IRQ4  �  COM2 = IRQ3  �  COM3 = IRQ4  �  COM4 = IRQ3  �

�               �               �               �               �

���������������������������������������������������������������;

Valid values are  /I3  and  /I4  (for IRQ3 and IRQ4).

�����������������������������������������������������������������������������

Command:  MODEM /L port command

Example:  MODEM /L 1 TERM

Action:   Writes all text from the modem to a file named MODEMLOG.TXT.

This is useful for saving the text of your session with your modem,

for later analysis.  The size of the LOG file is limited to 64K.

It is always created in the current directory.  The previous .TXT

file (if it exists) is renamed to .BAK.

Limits:   Most useful with TERM, TRACE and AT commands.  In TRACE mode,

Raise/Lower DTR is logged at [DTR+] and [DTR-] respectively.

�����������������������������������������������������������������������������

������������������������������������������������������������������������������

۱�� DO YOU HAVE ANY COMMENTS OR QUESTIONS?  You can reach us as follows: ۰��

۰��                                                                      ۱��

۱�� Voice Support Line (9 AM to 9 PM Eastern Time) ������ (514) 345-9578 ۰��

۰��                                                                      ۱��

۱�� 24 hr/day FREE files BBS (up to 9600 baud v42bis) ��� (514) 345-8654 ۰��

۰��                                                                      ۱��

۱�� GEnie ���������� T.CAMPBELL11       CompuServe ���������� 70154,1577 ۰��

������������������������������������������������������������������������������