Copyright (c) 1994 by K.H. Weiss  All rights reserved. Revision 3/9/94

Written by:

K.H. Weiss           Internet: <khweis1@mvmhp.ciw.uni-karlsruhe.de>

Eulenweg 2           Phone   : (49)-721-608-2418

76536 Weingarten               (49)-7244-1792

Germany

comtool

-------

This is a new version of my comtool program. This version has

hardwarehandshake built in. The local buffer is increased

to 1kB now. This fixed the problems with lost characters

when using 57600bps or 115.200bps under windows. I added a

new command wich allows you to enable/disable the fifo if

you have a serial port with a 16550 uart (^Fn)

I wrote 'comtool' because I needed a small program for testing and

commanding my serial ports and my modem. I use it for example to clear

the uart, set the baudrate and the control-lines of my modem or send some

'AT'- command sequences with it. You can even use it as a simple terminal

programm. (hint: install an ANSI-driver, nnansi is a good one).

You may invoke comtool either by defining portadress and hardware interrupt

or by writing only the number of comport (using a 'standard'-port)

comtool <Port-address> <irq> <Commands> or

comtool <number_of_ComPort> <Commands>

Each command begins with a 'caret' like this one ^. If there is no caret,

the following text will be send 'as is'.

Some Commands:

^fn    Enable/disable Fifo (if you have a 16550 uart). The hexadecimal

value n is written to the FCR (baseaddress + 2).

n=0   disable fifo

n=1   enable fifo, triggerlevel is 1

n=41               triggerlevel is 4

n=81               triggerlevel is 8

n=c1               triggerlevel is 14

^tn    wait n/10 seconds

^cr    send cr-character

^lf    send lf-character

^sp    send space-character

^^     send 'caret'

^d1    set dtr on

^d0    set dtr off

^h     hangup

^bn    set baudrate, n=0 115200

n=1 57600

n=2 38400

n=3 19200

n=4 9600

n=5 4800

n=6 2400

n=7 1200

^kn    send break-character, duration is n/10 seconds

^CDn   wait n/10 seconds for carrier-detect-signal. Set errorlevel 8 if

time over or user breaked, otherwise set errorlevel to 4

^wn string1 string2 string3

wait n/10 seconds for string1 (or 2 or 3), set the errorlevel

to 1, 2 or 3 if found or 8 if user breaked or time was over

^0n    send character with asci-value n. n is a hexadecimal expression < 256

Special keys:

Alt-H  Hangup the modem (may not work on all modems)

Alt-E  Exit comtool (may not work with all keyboards)

Ctrl-Break (or Ctrl-C) Breaks comtool (if Alt-E does'nt work)

new in comtool:

Alt-S Stops/Resumes transfer of data. The RTS handshakesignal is toggled.

(The modem must be programmed for hardwarehandshake of course)

Don't use the '<' - characters on commandline, because this is

used for redirection. That means: you can create a comandfile with

one or more comandlines in it and redirect this to comlog with '< cmdfile'

on the commandline.

If the last charcter on the commandline is NOT a caret, then you are in the

terminal-mode. You can type AT-sequences, connect to a site with 'atdt ...'

or do whatever you want. You leave this mode with Alt-E and if this doesn't

work (a keyboard-problem, one day I'll fix it ;-), then try Ctrl-C or

Ctrl-Break. If the last Character was a caret, then the programm returns to

DOS automatically. You can recall this programm whenever you want:

All Interrupts and the uart-state are restored carefully. You may shellout

from an other communication-application, start comlog, do something with it

and return to your application without any effect on your application.

It work's even with windows ...

Errorlevel

----------

The bit relevant errorlevel delivered by 'COMTOOL', means:

8          comtool breaked (ctrl-c, alt-e...)

4          carrier detected by the ^CD- command

1,2 or 3   one of three strings in datastream detected (^W -command)

example for an autodial batch (comport 1 used):

@echo off

:rep

COMTOOL 1 ^B2 +++ ^T6 atZ ^CR atdp072441792 ^CR ^W600 CONNECT BUSY NO^

if errorlevel 8 goto ende

if errorlevel 3 goto ende

if errorlevel 2 goto rep

if errorlevel 1 goto termi

goto rep

:termi

COMTOOL 1

:ende

COMTOOL ^H ^

------------------------------------------------------------------------

**********************************************************************

*  This program executable, help file, and related text files may be *

*  distributed freely and used without fee by any individual for     *

*  non-commercial use. This version of COMTOOL may  NOT be           *

*       distributed as part of any commercial package.               *

*                                                                    *

*           USE THIS SOFTWARE AT YOUR OWN RISK                       *

**********************************************************************

Report bugs and comments to             khweis1@mvmhp.ciw.uni-karlsruhe.de

--------------------------------------------------------------------------